# FreeRTOS

FreeRTOS is a popular free real-time operating system widely used with microcontrollers and small microprocessors. FreeRTOS is lightweight and open source. In addition to real-time applications, FreeRTOS is also suitable for low power, always-on and extremely fast boot applications.

https://www.freertos.org

# FreeRTOS on the Cortex-M4 of a Colibri iMX7

The NXP/Freescale i.MX 7 SoC which is the core of the Colibri iMX7 module implements a heterogeneous asymmetric architecture. Besides the main CPU core(s) based on the ARM Cortex-A7 processor, a secondary general purpose ARM Cortex-M4 core is available too. The secondary core typically runs a RTOS optimized for microcontrollers or a bare-metal application. Toradex provides FreeRTOS, a free professional grade real-time operating system for microcontrollers, along with drivers and several examples which can be used on our Colibri iMX7 platform. The FreeRTOS port is based on NXP FreeRTOS BSP for i.MX 7.

# Cortex-M4 Overview

The Cortex-M4 CPU core lives side by side with the Cortex-A7 based primary CPU cores. Both CPU complexes have access to the same interconnect and hence have equally access to all peripherals (shared bus topology). The graphic below is an incomplete and simplified drawing of the architecture with emphasis on the relevant sub systems to understand the heterogeneous asymmetric multicore architecture.

There are several types of memory available. The Cortex-M4 provides local memory (Tightly Coupled Memory, TCM), which is relatively small but can be accessed by the CPU without any latency. There are multiple OCRAM areas (On-Chip RAM, typically SRAM) which are relatively fast as well and slightly larger. The third option is the DDR3 based main memory. From a performance perspective one of the internal areas should be selected whenever possible.

A traditional microcontroller typically has internal NOR flash where the firmware is stored and executed from. This is not the case on Colibri iMX7: There is no NOR flash where the firmware can be flashed onto. Instead, the firmware needs to be stored on the mass storage device such as SD-card or the internal NAND flash. The available mass storage devices are not "memory mapped", and hence application cannot be executed directly from any of the cores (no eXecuted-In-Place, XIP). Instead, code need to be loaded into one of the available memory sections before the CPU can start executing it.

The i.MX 7 SoC always boots using the Cortex-A7 core. The core executes the internal boot ROM which typically loads a boot loader such as U-Boot. The boot loader allows loading the firmware from the mass storage device (e.g. NAND flash) into memory and triggers the Cortex-M4 to start executing the firmware. To upgrade or replace a firmware, one can just replace the firmware binary on the mass storage device.

## Memory areas

The two CPU platforms use a different memory layout to access individual sub systems. This table lists some important areas and their memory location for each of the cores side by side. The full list can be found in the i.MX 7 reference manual.

## Resource Domain Controller

The RDC prohibits and grants access to peripherals and memory areas for individual bus masters (e.g. CPU, DMA controller) on hardware level. The RDC allows to define up to 4 resource domains and assign peripherals and memory locations to those resource domains. By default, the A7 core is in domain 0 and all peripherals are assigned to the domain 0. When the FreeRTOS firmware start, the Cortex-M4 core is in domain 0 too, but then reassigns the Cortex-M4 and the required peripherals to domain 1 (see board.c and the example specific hardware_init.c).

If a device shall be used on the Cortex-M4 which is used by the Linux kernel running on the Cortex-A7 (e.g. I2C), it is important to disable this device in the device tree of the Linux kernel (e.g. set the status property to disabled). The article Device Tree Customization explains in more details how to alter the device tree.

# Setup Software

## Update current Linux distribution,

$ sudo apt update $ sudo apt upgrade

## Install 32-bit version of libc and libncurses

$ sudo dpkg --add-architecture i386 $ sudo apt-get install libc6:i386 libncurses5:i386

## Install GNU make and cmake

$ sudo apt-get install make cmake build-essential git

## Install Linaro provided ARM Embedded toolchain:

$ cd ~ $ wget https://launchpad.net/gcc-arm-embedded/4.9/4.9-2015-q3-update/+download/gcc-arm-none-eabi-4_9-2015q3-20150921-linux.tar.bz2 $ tar xjf gcc-arm-none-eabi-4_9-2015q3-20150921-linux.tar.bz2

## Get the FreeRTOS Source Code

$ git clone -b colibri-imx7-m4-freertos-v8 git://git.toradex.com/freertos-toradex.git freertos-colibri-imx7/ $ cd freertos-colibri-imx7

## Build an example

The scripts expect the environment variable to point to the Linaro ARM Embedded toolchain:

$ export ARMGCC_DIR=~/gcc-arm-none-eabi-4_9-2015q3/ $ cd examples/imx7_colibri_m4/demo_apps/hello_world/armgcc $ ./build_all.sh

With that, the sub directories release/ and debug/ contain the firmware binaries which can be executed on the target.

Note on VirtualBox:

If you are using virtual machine in VirtualBox, be sure to install the guest additions CD:

# Setup Hardware

VGA

## Connect power supply with 12 V to barrel jack X35 or terminal block connector X33 .

## Connect a VGA monitor to X24.

## Connect an Ethernet cable to X17.

## Connect a USB Type-B to Type-A cable to X27.

## Connect a Serial RS-232 Cable to UART B (Top) X25

## Connect Segger Jlink to X13

## Turn on the board.

# Setup U-Boot

## Add your user to dialout group

$ sudo adduser <your_user_name> dialout

## Install Putty program

$ sudo apt install putty

- Find out the serial port connected to your Linux host.

With the Toradex board connected and turned on as in the Setup Hardware section, use the next command, the port should be something like “ttyUSB0”

$ dmesg ¦ grep tty

## Open Putty program and configure with the found serial port.

$ putty

Select the “Serial” option in the left and make sure the parameters are configured the same, replace the port with the one you found in your machine.

In the main “Session” section, select Serial and press connect:

## Enter U-Boot

The screen will be black, press “Enter” to see the Linux asking for user login, do no enter nothing, reset the board, and quickly in the next screen press any key to enter U-Boot.

## Configure U-Boot with the next commands

Colibri iMX7 # setenv defargs clk_ignore_unused Colibri iMX7 # setenv fdt_fixup 'fdt addr ${fdt_addr_r} && fdt rm /soc/aips-bus@30800000/spba-bus@30800000/serial@30890000' Colibri iMX7 # saveenv

Linux disables unused clocks by default. However, Linux is not aware what clocks are used by the Cortex-M4 core, therefore one should use the clk_ignore_unused kernel parameter.

By default, our Linux device tree uses UART_B too, which leads to a external abort when the Linux kernel tries to access UART_B. It is recommended to alter the device tree and disable UART_B using the status property. Temporary, the following fdt_fixup command can be use in U-Boot

## Set the auto start to the Cortex-M4 application

Colibri iMX7 # setenv m4boot 'fatload mmc 0:1 ${loadaddr} hello_world.elf && bootaux ${loadaddr}' Colibri iMX7 # saveenv Colibri iMX7 # reset

## Copy the example application to the boot partition of the Toradex Board

Once Linux is started, on the Toradex board login with ‘root’ user, then find out the IP address of the board with:

# ifconfig

Create a mount point and mount the first eMMC partition:

$ mkdir /mnt/vfat_partition $ mount /dev/mmcblk0p1 /mnt/vfat_partition

Now on your Linux host (virtual machine) copy the example you built in: Build an example to the Toradex Board, be sure to copy the.elf extension

$ scp hello_world.elf root@<board_ip>:/mnt/vfat_partition/.

Back again at the Toradex Board, unmount the partition:

$ umount /mnt/vfat_partition

Reset your board, setting up to auto start the Cortex-M4 application will make easier to debug a new application.

# Install and Configure Eclipse

## Install Eclipse

In the Linux host PC download Eclipse from here:

https://www.eclipse.org/downloads/packages/

Select the Linux-64-bit version:

Open terminal and move to the path where the file was downloaded and extract it

$ cd ~/Downloads $ tar -xzf eclipse-cpp-2019-09-R-linux-gtk-x86_64.tar.gz $ sudo apt install default-jre

## Start Eclipse

To run eclipse execute:

$ ~/Downloads/eclipse/eclipse

Select the workspace and press Launch:

Close Welcome page If open.

## Import Project

In the “Project Explorer Panel”, import example project

Select “Existing Project”

Select the path folder where the hello world program is located and press “Open”

It should be like “freertos-colibri-imx7/examples/imx7_colibri_m4/demo_apps/hello_world/armgcc”

If more than one Project is shown, select only the one that ends with “armgcc/debug”, then press “Finish”

## Install Eclipse Plugin

Help -> Install New Software

Add the URL: http://gnuarmeclipse.sourceforge.net/updates

Select and press Next to install and follow the instructions on screen.

## Install JTAG tools

Download “J-Link Software and Documentation pack for Linux, DEB installer, 64-bit” from:

https://www.segger.com/downloads/jlink/JLink_Linux_x86_64.deb

Locate where the file was downloaded, then in a terminal execute

$ sudo dpkg -i ~/Downloads/JLink_Linux_V652e_x86_64.deb $ sudo apt-get install -f

## Debugger Configuration

Enter to Run -> Debug Configurations…

Select “GDB SEGGER -J-Link Debugging” and click on “New” button.

Selecting the new configuration and In the  Tab make sure to have the next configuration:

J-Link GDB Server Setup section

| Device name: | MCIMX7D7_M4 |
| --- | --- |
| Interface: | JTAG |
| Other options: | -endian little -scriptfile /home/isaias/toradex/imx7/Toradex_iMX7D_Connect_CortexM4.JLinkScript -nosilent |

Download the JLinkScript file from here: Toradex_iMX7D_Connect_CortexM4.JLinkScript

GDB Client Setup section

Executable: /home/isaias/gcc-arm-none-eabi-4_9-2015q3/bin/arm-none-eabi-gdb

Important: Be sure to replace the path names, with your corresponding path names.

In  Tab, add “monitor reset 0” to the “Initialization Commands”

Finally click on “Apply”

## Debugging Application

Now we are ready to debug the application, press Debug.

# Running a Firmware on Cortex-M4

| Copy from SD card to memory | fatload mmc 1:1 ${loadaddr} <app_name>.elf |
| --- | --- |
| Copy from eMMC to memory | fatload mmc 0:1 ${loadaddr} <app_name>.elf |
| Copy from tftp to memory | tftp ${loadaddr} <app_name>.elf |
| Run app | bootaux ${loadaddr} |
| Get dynamic IP address | dhcp |
| Set TFTP server IP address | setenv serverip <ip_address> |

## Store a Firmware on Flash and Run it on Boot

For the Colibri iMX7 1GB, which features eMMC storage, you may just write the M4 firmware to the VFAT partition alongside the Linux kernel and device tree and load it from there.

First on the Colibri Board, create a mount point and mount the first eMMC partition:

# mkdir /mnt/vfat_partition # mount /dev/mmcblk0p1 /mnt/vfat_partition

Then, from your Linux host (Virtual Machine), copy your .elf file to that directory:

$ scp hello_world.elf root@<board_ip>:/mnt/vfat_partition

Finally, at Colibri Board unmount the partition:

root@colibri-imx7-emmc:/# umount /mnt/vfat_partition

Reset your board, access the U-Boot terminal and update the m4boot environment variable:

Colibri iMX7 # setenv m4boot 'fatload mmc 0:1 ${loadaddr} hello_world.elf && bootaux ${loadaddr}' Colibri iMX7 # saveenv

For the gpio_bank2_imx example, the Switch 1 goes to SODIMM 133 and the Led1 to SODIMM 127

# CAN communication example

## Set up the Colibri Evaluation Board

Review the Colibri iMX7 SoC Datasheet  at section 5.24 Controller Area Network (CAN), the table 5-46 shows the CAN signal pins.

Compare it with the Colibri Evaluation Board PDF Schematics  at page 19 where there is the X2 connector schematic.  At the left of the page, it’s the pinout of SoC that is connected to the board:

The conclusion is that the Colibri iMX7 RX and TX signals are not connected to the CAN controller and transceiver that end at X2 connector shown in page 19 of the Evaluation Board datasheet.

But in the same page 19 schematic, it is shown that jumpers JP4 and JP5 can be used to disconnect the CAN controller and using X38 connector CAN_TX and CAN_RX signals can be connected directly to the Digital Isolator and the it will go to the CAN transceiver then to the X2 connector.

Now, we need to find out where pins shown in table 5-46 are connected. At page 9 of the Colibri Evaluation Board Datasheet, there is the schematic that shows where are those pins connected

Finally, now we know what is needed:

- Open JP4 y JP5

- Connect the next signals:

|  | Top X2 connector | X38 connector |  |
| --- | --- | --- | --- |
| SODIM_55 | 5 | 1 | Tx |
| SODIM_63 | 6 | 2 | Rx |

In this way the iMX7 CAN controller can be connected to the CAN transceiver and then the output will go to X2 connector.

## Software example

### Compile the flexcan example from the previously downloaded FreeRTOS code.

$ export ARMGCC_DIR=~/gcc-arm-none-eabi-4_9-2015q3/ $ cd examples/imx7_colibri_m4/ driver_examples/flexcan/flexcan_network/armgcc/ $ ./build_all.sh

### Import the project to eclipse, see Import Project from Install and Configure Eclipse section

- Create a Debugger configuration, see Debugger Configuration from Install and Configure Eclipse

- Debug the application

In order to see CAN communication, connect a valid CAN network at the Top X2 connector.

## Set up Linux device tree

### Download to your linux PC, the new device tree from here

### Connect to the Linux serial console with conector X27 (see Setup Hardware)

### Turn on the Colibri board, make sure is connected to Ethernet and let Linux start

### Open a Putty session in the corresponding serial port and press Enter to start the console

### Log in as ‘root’ user and find out the address of the Colibri board with the command:

# ifconfig

Example:

### Create a mount point and mount the first eMMC partition:

# mkdir /mnt/vfat_partition # mount /dev/mmcblk0p1 /mnt/vfat_partition

### Back at the your Linux PC, we will copy the new device tree to the Cilibro Board with the IP found in the previous steps and the command:

# scp imx7d-colibri-emmc-eval-v3.dtb root@172.17.44.54:/mnt/vfat_partition/.

### Once the dtb file is copied, return to the Putty console and restart the Colibri board

# reboot

## Diff in device tree file

- Disable mcp2515 SPI CAN controller

- Disable multiplexing of the pins SODIMM 55 and SODIMM 63

diff --git a/arch/arm/boot/dts/imx7-colibri-eval-v3.dtsi b/arch/arm/boot/dts/imx7-colibri-eval-v3.dtsi old mode 100644 new mode 100755 index d9526853bdc0..efee588b3b1c --- a/arch/arm/boot/dts/imx7-colibri-eval-v3.dtsi +++ b/arch/arm/boot/dts/imx7-colibri-eval-v3.dtsi @@ -139,7 +139,7 @@ interrupt-parent = <&gpio5>; interrupts = <2 IRQ_TYPE_EDGE_FALLING>; spi-max-frequency = <10000000>; - status = "okay"; + status = "disabled"; }; spidev0: spidev@0 { diff --git a/arch/arm/boot/dts/imx7-colibri.dtsi b/arch/arm/boot/dts/imx7-colibri.dtsi old mode 100644 new mode 100755 index 4a1ef3f83faf..395973628f27 --- a/arch/arm/boot/dts/imx7-colibri.dtsi +++ b/arch/arm/boot/dts/imx7-colibri.dtsi @@ -361,7 +361,7 @@ &iomuxc { pinctrl-names = "default"; pinctrl-0 = <&pinctrl_gpio1 &pinctrl_gpio2 &pinctrl_gpio3 &pinctrl_gpio4 - &pinctrl_gpio5 &pinctrl_gpio6 &pinctrl_gpio7>; + &pinctrl_gpio5 &pinctrl_gpio6>; pinctrl_gpio1: gpio1-grp { fsl,pins = <

## Compilation commands

sudo apt install gcc-arm-linux-gnueabihf device-tree-compiler export ARCH=arm export CROSS_COMPILE=arm-linux-gnueabihf- make colibri_imx7_defconfig make imx7d-colibri-emmc-eval-v3.dtb

## Reference

Freescale IOMUX Controller (IOMUXC) for i.MX

iMX7 U-Boot Versions

FlexCAN on Colibri iMX7

# Communicating between cores with RPMsg

## Set up Linux

On the Toradex development board

### Connect to the Linux serial console with conector X27 (see Setup Hardware)

### Turn on the board and let Linux start

### Open a Putty session in the corresponding serial port and press Enter to start the console

### Log in as ‘root’ user and enter the command

# echo imx_rpmsg_tty > /etc/modules-load.d/rpmsg_tty.conf

This command will load the imx_rpmsg_tty module automatically on every boot

## Set up demo application

### At your Linux development PC, clone or update the git repository with:

# git clone https://github.com/SergioEspinoza/SemestreI_IMX7

This will create a new SemestreI_IMX7 folder inside there is an example application

### Copy the demo_semestre_i folder to the freertos-colibri-imx7 directory where you have been working on, example:

# cp -r SemestreI_IMX7/Colibri_iMX7_Cortex-M4/demo_semestre_i ~/toradex/imx7/freertos-colibri-imx7/examples/imx7_colibri_m4/demo_apps/.

In your PC, use the correct paths.

### Locat at the demo_semestre_i directory and execute the build command:

# cd ~/toradex/imx7/freertos-colibri-imx7/examples/imx7_colibri_m4/demo_apps/demo_semestre_i/armgcc # export ARMGCC_DIR=~/gcc-arm-none-eabi-4_9-2015q3/ # ./build_all.sh

### Import the project to Eclipse, see: Import Project and configure Debug, see: Debugger Configuration

## Sequence to run the application

The correct order to make work the application is:

### See section Setup Hardware

### Start a Linux console with Putty/minicom on  the serial port of the Toradex Board (X27 USB connector), see Open Putty program and configure with the found serial port.

### Start another Putty/minicom serial M4 console (X25 DB9 connector),

### Turn on the toradex board

### Stop Linux from starting, in order to start U-Boot (see: Enter U-Boot)

### At your Linux development PC, debug the demo_semestre_i application with Eclipse, see: Debugging Application you should see somehting like this at yout M4 console

### Once the application started debbuging, at the serial console on U-Boot execute the command “boot” to start.

### At the M4 console, you should see a handshake message:

### Once Linux is started, login with “root”, to send a message execute:

# echo Test message >/dev/ttyRPMSG0

To star again the debug again the board needs to be reset, then start again at step 5.

Communicating between cores with RPMsg

Change size of RPMsg buffers

How to change the size of RPMSG buffer

RPMsg PingPong Example with Boot2Qt Stack

# Reference

- FreeRTOS on the Cortex-M4 of a Colibri iMX7

- First steps with Heterogeneous Multicore Processing on the NXP i.MX 7

- CAN on Colibri i.MX7

- Colibri iMX7

- Colibri Evaluation Board

- https://github.com/SergioEspinoza/SemestreI_IMX7